End of Sailing Week/Weekend Checklist

- Plumbing:

  - Shut off Stove valves and CNG cylinder

  - Closes all through hull valves:

    - Engine:  Leave tag on nav station control panel

    - Head:  Check both through hull valves

    - Galley:  Sink Drain

  - Turn off shore water, if connected

    - Close valve on inline filter

    - Drain water by turning on water in galley

    - Turn off water on dock and vent hoses

- Electrical:

  - Turn off all DC appliances and lights in boat

  - Turn off all DC circuit breakers except “cabin”

  - Turn Bilge pump switch to “Auto”

  - Turn off inverter

  - Turn off Hot Water Heater

  - Turn off AC switch

  - Turn of VHF radios

  - Disconnect the AC cord on the dock if connected.

  - Leave battery switch on “1”

- Hatches:

  - Closes all hatches

    - Secure all screens on hatches

    - Forward hatch

    - 2 hatches in head

    - 2 forward starboard hatches

    - Hatch over dinette

    - Hatch in aft cabin

- Cockpit:

  - Remove gear from cockpit e.g. blocks, winch handles and lines

  - Remove cockpit awning

  - Remove all cushions from cockpit

  - Close dodger window

  - Lock both cockpit locker hatches

- Flags:

  - Remove any burgees

  - Remove and roll up the American Flag on the flag staff on the stern

- Remove gear from boat:

  - Unload refrigerator; drain and clean as necessary

  - Bag and remove garbage

  - Bag and remove recyclables

  - Bag and remove empty beverage cans or bottles

  - Remove bedding if necessary

  - Remove clothing in laundry bags

  - Remove duffle bags

  - Remove extra food as necessary

  - Remove cell phones, and cameras

  - Remove wallets and purses and car keys

  - Remove boat keys

  - Remove all eye glasses

- Check dock lines:

  - Tighten Port lines to insure boat is not up to the dock

  - Review all cleats to insure properly secured

  - Close and lock dock box

- Companionway Hatch:

  - Close screen before installing drop boards